LED Blinking mikroC

- Click on Project >> New Project

- Click Next

- Enter Project name, path (created folder path), clock frequency, microcontroller and click Next. Clock Frequency is the frequency of oscillator used with microcontroller. Here we use PIC 16F877A microcontroller with 8MHz crystal.

- Here you can add your subprogram files or user defined header files in large projects. Hence we are dealing with simple LED Blinking in this tutorial, ignore it and click Next.

- Click Finish, to complete the New Project Wizard.

- Then you will see the editor, where you can enter the MikroC Code.

# MikroC Programming

Before going to the programming you should understand the following things:

- Output pins of a PIC Microcontroller is divided in to different PORTS containing a group of GPIO (General Purpose Input Output Pins) pins.

- In 16F PIC Microcontrollers, there are two registers associated with a port, TRIS and PORT. eg: TRISB, PORTB, TRISC, PORTC

- TRIS stands for Tri-State, which determines the direction of each GPIO pin. Logic 1 at a particular bit of TRIS register makes the corresponding pin Input and Logic 0 at a particular bit of TRIS register makes the corresponding pin Output. An Input pin of PIC Microcontroller have very high input impedance, thus it may said to be in Hi-Impedance state.

- PORT register is used to read data from or write data to GPIO pins. Logic 1 at a particular bit of PORT register makes the corresponding pin at Logic High (VDD) and Logic 0 at a particular bit of PORT register makes the corresponding pin at Logic Low (VSS) if that pin is an Output pin (TRIS bit is 0).

- PORT register can be used to read digital data from an Input pin. Logic 1 indicates the pin is at Logic High and Logic 0 indicates that the pin is at Logic Low.

- You can write to PORT and TRIS register entirely or bit by bit.

TRISC.F0 = 1; 	//Makes 0th bit of PORTC Input

TRISC.F5 = 0; 	//Makes 5th bit of PORTC Output

PORTB.F3 = 1; 	//Makes 3ed bit of PORTB at Logic High

PORTB.F7 = 0; 	//Makes 7th bit of PORTB at Logic Low

- A number with a prefix ‘0b’ indicates a binary number.

- A number with a prefix ‘0’ indicates an octal number.

- A number with a prefix ‘0x’ indicates a hexadecimal number.

- A number without prefix is a decimal number.

Let’s see some examples…

| Decimal | Binary | Octal | Hexadecimal |
| --- | --- | --- | --- |
| 0 | 0b00000000 | 00 | 0x00 |
| 1 | 0b00000001 | 01 | 0x01 |
| 128 | 0b10000000 | 0200 | 0x80 |
| 255 | 0b11111111 | 0377 | 0xFF |

PORTB = 0xFF; //Makes all pins of PORTB Logic High

TRISC = 0x00; //Makes all pins of TRISC Output

PORTD = 128; //Makes 7th bit of PORTD Logic High

MikroC Code – Blinking an LED

The following program blinks an LED with a delay of 1 second.

void main()

{

TRISB.F0 = 0; //Makes PORTB0 or RB0 Output Pin

while(1) //Infinite Loop

{

PORTB.F0 = 1; //LED ON

Delay_ms(1000); //1 Second Delay

PORTB.F0 = 0; //LED OFF

Delay_ms(1000); //1 Second Delay

}

}

- Enter the above MikroC code

Save it

Then Compile it. Click Build >> Build (or Ctrl+F9)

A hex file will be generated in your Project Folder. You need to write this file to microcontroller using a programmer.