- Generate a Sequence of Blinking LEDs using PIC microcontroller Click on Project >> New Project

- Click Next

- Enter Project name, path (created folder path), clock frequency, microcontroller and click Next. Clock Frequency is the frequency of oscillator used with microcontroller. Here we use PIC 16F877A microcontroller with 8MHz crystal.

- Here you can add your subprogram files or user defined header files in large projects. Hence we are dealing with simple LED Blinking in this tutorial, ignore it and click Next.

- Click Finish, to complete the New Project Wizard.

- Then you will see the editor, where you can enter the MikroC Code.

Exercise – Task

If the pushbutton on RB1 is pressed the LED blinking sequence would be initiated.

Pull-up resistors on PORTB

Configure OPTION register to enable pull up resistors on port B

RBPU – Port B Pull up Enable bit

1- Porta B pull-ups are disabled

0 – Porta B pull-ups are enabled

Three different ways to enable Pull-up resistors:

OPTION.F7 =0; // set individual bit RPBU

OPTION_REG= 0b00000000; // set OPTION_REG as byte binary

OPTION_REG= 0x00; // set OPTION_REG as byte in hex form

TRISB register

MikroC Code – Blinking an LED

The following program blinks an LED with a delay of 1 second.

void main()

{

TRISB.F0 = 0; //Makes PORTB0 or RB0 Output Pin

while(1) //Infinite Loop

{

PORTB.F0 = 1; //LED ON

Delay_ms(1000); //1 Second Delay

PORTB.F0 = 0; //LED OFF

Delay_ms(1000); //1 Second Delay

}

}

- Enter the above MikroC code

Save it

Then Compile it. Click Build >> Build (or Ctrl+F9)

A hex file will be generated in your Project Folder. You need to write this file to microcontroller using a programmer.